STCD (STation Coordinates Difference) format

(version 1.0)

L. Soudarin, M. Feissel-Vernier, J.J. Valette

Introduction.

The STCD format has been developed by the IDS Central Bureau and the Analysis Coordinator with the objective to provide a user-friendly presentation of DORIS coordinate time-series results for a potentially broader community of users.

The STCD files are ASCII files giving for one geodetic point the times series of both Cartesian and geographic coordinates. Coordinate time series consist of residuals relative to a reference position. Each coordinate of the series is assumed to be expressed in a same reference system at the observation epoch. Users can then easily plot the time evolution of the point position and observe its horizontal and vertical displacement.

The STCD file consists of 5 mandatory blocks. 4 blocks derived of the SINEX format (FILE/REFERENCE; FILE/COMMENT; SITE/ID; SOLUTION/APRIORI) form the header. They provide information needed to the creation of the series of which main elements are the reference position of the geodetic point, and the characteristics of the ellipsoid used to express the Cartesian coordinates in geographic system. The 5th block gives the coordinate differences as a function of time. Time unit is the Modified Julian Day (MJD). Residual unit is the millimetre.

We propose hereafter a description of the STCD format, and an example of a STCD file.

Note that STCD files of DORIS stations can easily be generated from a series of SINEX files, under the condition that the SINEX solutions are all expressed in a common reference frame.

Tools to extract STCD from a series of SINEX files are currently being developed by the Central Bureau.

Description.

The 4 SINEX-like blocks follow the general description rules of the SINEX format (Blewitt 2003). They are formed of line of 80 characters or less. The 5th block (coordinate differences) is a 13-column block of which fields and format are described in the FILE/COMMENT block.

The blocks are separated by lines of 79 “-“ starting with “*”. There is no such a line after the 5th block. The total of the lines of the 4 blocks including the separating lines is exactly 29. The first line of the coordinate differences block is the 30th.

Block #1

FILE/REFERENCE Block (Mandatory)

Description:

This block provides description of the input and output, information on the organization, point of contact, the software and hardware involved in the creation of the file.

Contents:

DESCRIPTION	Name of Analysis Center

OUTPUT		Description of the file contents

CONTACT		Name and email of the relevant contact

SOFTWARE		Software use to generate the results

HARDWARE		Computer hardware on which above software was run

INPUT			Brief description of the input used to generate this time series

The block starts with the line

+FILE/REFERENCE

The block ends with the line

-FILE/REFERENCE

Block #2

FILE/COMMENT Block (Mandatory)

Description:

This block provides information on the content of the block Coordinate Differences (fields, format, units), reference system in which the coordinates are expressed, and description of the Earth ellipsoid (flattening factor and equatorial radius) used to convert the Cartesian coordinates into geographical coordinates.

Contents:

FIELDS - modified julian date, dX, dY, dZ, sX, sY, sZ, dEast, dNorth, dUp, sEast, sNorth, sUp

FORMAT - 2x,f7.1,2(2x,3(1x,f6.1),3(1x,f5.1))

UNITS - all position residuals in millimeters

REFERENCE SYSTEM – Name of the reference system

EARTH ELLIPSOID - flattening factor: value  equatorial radius: value unit

The block starts with the line

+FILE/COMMENT

The block ends with the line

-FILE/COMMENT

Block #3

SITE/ID Block (Mandatory)

Description:

This block provides general information for the site containing estimated parameters: site code, point code, DOMES number, station description, approximate longitude, approximate latitude, approximate height

Contents:

Comment line (mandatory)

*Code Pt __Domes__ T _Station Description__ _Longitude_ _Latitude__ _Height

immediatly followed on one line by the parameters described in the table below:

[Site Code] [Point Code] [DOMES number] [Observation Code] Station description Approximate Longitude Approximate Latitude Approximate Height

The block starts with the line

+SITE/ID

The block ends with the line

-SITE/ID

___________________________________________________________________

|_________________S_I_T_E___I_D___D_A_T_A___L_I_N_E_________________|

|                |                                   |              |

|__Field_________|______Description__________________|___Format_____|

|                |                                   |              |

| [Site Code]    |4 character mnemonic of the station|   1X,A4      |

|________________|___________________________________|______________|

|                |                                   |              |

| [Point Code]   | Physical monument used at the site|   1X,A2      |

|________________|___________________________________|______________|

|                |                                   |              |

| [DOMES number] | Unique alpha-nummeric monument    |   1X,A9      |

|                | identification. For ITRF purposes,|              |

|                | it is a 9 character DOMES number  |              |

|                | (5 digits, followed by the single |              |

|                | letter 'S', followed by 3 digits) |              |

|________________|___________________________________|______________|

|                |                                   |              |

| [Observation   | Observation technique(s) used.    |   1X,A1      |

| Code]          | Code for DORIS is D               |              |

|________________|___________________________________|______________|

|                |                                   |              |

| Station        | Free-format description of the    |   1X,A22     |

| description    | site, typically the town and/or   |              |

|                | country.                          |              |

|________________|___________________________________|______________|

|                |                                   |              |

| Approximate    | Approximate longitude of the site |   1X,I3,     |

| Longitude      | in degrees(E/+), minutes and      |   1X,I2,     |

|                | seconds.                          |   1X,F4.1    |

|________________|___________________________________|______________|

|                |                                   |              |

| Approximate    | Approximate latitude of the site  |   1X,I3,     |

| Latitude       | in degrees(NS/+-), minutes and    |   1X,I2,     |

|                | seconds.                          |   1X,F4.1    |

|________________|___________________________________|______________|

|                |                                   |              |

| Approximate    | Approximate height of the site in |   1X,F7.1    |

| Height         | metres.                           |              |

|________________|___________________________________|______________|

|              |

|     75       |

|______________|

Block #4

SOLUTION/APRIORI Block (Mandatory)

Description:

Reference position with respect to which residuals are given

Contents:

Comment line (mandatory)

*Index _Type_ Code Pt Soln _Ref_Epoch__ Unit S __Estimated Value____ _Std_Dev___

immediatly followed on three lines by the station coordinates X,Y,Z as described in the table below:

Parameter index, parameter type, site code, point code, solution ID, time, parameter units, constraint code, parameter value, parameter standard deviation.

The block starts with the line

+ SOLUTION/APRIORI

The block ends with the line

- SOLUTION/APRIORI

___________________________________________________________________

|________S_O_L_U_T_I_O_N___A_P_R_I_O_R_I___D_A_T_A___L_I_N_E________|

|                |                                   |              |

|__Field_________|______Description__________________|___Format_____|

|                |                                   |              |

| Parameter Index| Index of a priori parameters.     |   1X,I5      |

|________________|___________________________________|______________|

|                |                                   |              |

|[Parameter Type]| Identification of the type of     |   1X,A6      |

|                | parameter.                        |              |

|                | STAX - Coordinate in X-direction m|              |

|                | STAY - Coordinate in Y-direction m|              |

|                | STAZ - Coordinate in Z-direction m|              |

|________________|___________________________________|______________|

|                |                                   |              |

| [Site Code]    |4 character mnemonic of the station|   1X,A4      |

|________________|___________________________________|______________|

|                |                                   |              |

| [Point Code]   | Physical monument used at the site|   1X,A2      |

|________________|___________________________________|______________|

|                |                                   |              |

| [Solution ID]  | Solution ID at the Site code      |   1X,A4      |

|________________|___________________________________|______________|

|                |                                   |              |

| [Time]         | Epoch at which the apriori        |   1X,I2.2,   |

|                | parameter or the inner constraint |   1H:,I3.3,  |

|                | is valid.                         |   1H:,I5.5   |

|________________|___________________________________|______________|

|                |                                   |              |

| Parameter Units| Units used for the coordinates    |   1X,A4      |

|                | and sigmas. They should be m      |              |

|________________|___________________________________|______________|

|                |                                   |              |

| [Constraint    | Constraint applied to the parame- |   1X,A1      |

| Code]          | ter.                              |              |

|________________|___________________________________|______________|

|                |                                   |              |

| Parameter value| Value of the coordinate           |   1X,E21.15  |

|________________|___________________________________|______________|

|                |                                   |              |

| Parameter      | Standard deviation for the        |   1X,E11.6   |

| Standard       | coordinate                        |              |

| Deviation      |                                   |              |

|________________|___________________________________|______________|

|              |

|     80       |

|______________|

Block #5

The coordinate time series. No name block (Mandatory)

Description: coordinate differences in cartesian (XYZ) and geographic (East (E), North (N), Up (U) and associated uncertainties all expressed in mm as a function of time expressed in Modified Julian Day.

Contents:

One result per line, 13 fields separated by blanks:

MJD (Modified Julian Date), dX (X residual), dY (Y residual), dZ (Z residual), sigdX (uncertainty of dX), sigdY (uncertainty of dY), sigdZ (uncertainty of dZ), dE (East residual), dN (North residual), dU (Up residual), sigdE (uncertainty of dE), sigdN (uncertainty of dN), sigdU (uncertainty of dU)

Example of STCD file.

+FILE/REFERENCE

DESCRIPTION   LEGOS/CLS Analysis Center

OUTPUT        Monthly position residuals at observation epoch

CONTACT       Laurent Soudarin <laurent.soudarin@cls.fr> info-doris@cls.fr

SOFTWARE      GINS / DYNAMO (GRGS/CNES)

HARDWARE      SUN / CRAY

INPUT         SPOT2, SPOT3, SPOT4, SPOT5, TOPEX, ENVISAT DORIS data

-FILE/REFERENCE

*_______________________________________________________________________________

+FILE/COMMENT

FIELDS - modified julian date, dX, dY, dZ, sX, sY, sZ, dEast, dNorth, dUp, sEast, sNorth, sUp

FORMAT - 2x,f7.1,2(2x,3(1x,f6.1),3(1x,f5.1))

UNITS - all position residuals in millimeters

REFERENCE SYSTEM - ITRF2000 using a global LCA solution (1993_2004) for transformation

EARTH ELLIPSOID - flattening factor: 298.257810 equatorial radius: 6378136.0 m

-FILE/COMMENT

*_______________________________________________________________________________

+SITE/ID

*Code Pt __Domes__ T _Station Description__ _Longitude_ _Latitude__ _Height

AMSA  A 91401S001 C AMSTERDAM antenna       77 34 17.0 -37 47 54.3    62.3

-SITE/ID

*_______________________________________________________________________________

+SOLUTION/APRIORI

*Index _Type_ Code Pt Soln _Ref_Epoch__ Unit S __Estimated Value____ _Std_Dev___

1 STAX   AMSA  A    1 97:001:00000 m    2 0.108606165885490E+07 0.17099E-02

2 STAY   AMSA  A    1 97:001:00000 m    2 0.492796300849270E+07 0.89032E-03

3 STAZ   AMSA  A    1 97:001:00000 m    2 -.388782838175110E+07 0.89761E-03

-SOLUTION/APRIORI

*_______________________________________________________________________________

49001.9     -0.9   42.6   51.5  12.0   8.3  10.9     10.1   66.3    1.1  15.4   9.6   9.6

49031.2     -1.4   28.1   33.3  12.7   8.7  11.7      7.4   43.1    1.0  16.1  10.5  10.0

49064.0     -8.5    7.5   11.4  13.4   9.3  11.8      9.9   12.4   -2.6  17.1  10.5  10.5

49093.3     13.2   20.4   36.0  12.6   9.2  12.5     -8.5   42.5   -4.0  16.1  11.1  10.7

49126.1    -19.2   45.3   18.4  11.1   8.0  11.1     28.4   39.3   20.4  14.1  10.0   9.2

49155.3    -27.6   40.0   38.6  10.6   7.4  10.4     35.5   51.0    2.5  13.6   9.3   8.6

49184.6      9.3   13.7   21.4  11.3   7.9  10.7     -6.1   26.4   -1.0  14.3   9.5   9.3

49213.8     20.9   15.6   18.9  10.6   7.3  10.3    -17.1   27.1    4.0  13.5   9.2   8.5

49246.7    -38.5   22.1   35.1  11.3   7.5  10.4     42.3   36.0  -11.1  14.4   9.3   8.7

49275.9    -40.9   24.4   20.1  11.2   8.3  10.6     45.1   25.2   -0.4  14.3   9.3   9.6

49308.7    -17.6   28.9   32.2   9.2   6.3   8.9     23.3   40.5   -0.5  11.7   8.0   7.4

49338.0      4.2   28.1    8.2  10.1   6.7   9.1      1.9   23.9   17.4  12.8   8.2   7.7

49367.2    -25.8   42.8   14.2   9.7   6.5   9.2     34.4   33.5   20.0  12.4   8.3   7.5

49396.4    -20.4   37.6   10.8   9.4   6.4   8.9     28.0   28.4   18.9  12.0   8.0   7.4

49429.3    -29.4   31.3   23.3   8.6   5.8   8.4     35.4   33.4    4.9  10.9   7.5   6.9

49458.5     -9.2   25.5   14.0   8.4   5.6   8.5     14.5   25.2    9.5  10.7   7.6   6.8

49491.4    -17.9   28.3   -1.6  12.5   8.7   9.7     23.6   13.4   19.8  16.1   8.6   9.6

Annex

Relationship between MJD and decimal year.

Decimal year = 2000.0000 + (MJD-51544.03)/365.2422

Relationship between MJD and day of year (DOY):

----------------------------------------------

Year       MJD           Year       MJD

----------------------------------------------

1991  48256 + DOY        2006  53735 + DOY

1992  48621 + DOY        2007  54100 + DOY

1993  48987 + DOY        2008  54465 + DOY

1994  49352 + DOY        2009  54831 + DOY

1995  49717 + DOY        2010  55196 + DOY

1996  50082 + DOY        2011  55561 + DOY

1997  50448 + DOY       2012  55926 + DOY

1998  50813 + DOY        2013  56292 + DOY

1999  51178 + DOY        2014  56657 + DOY

2000  51543 + DOY        2015  57022 + DOY

2001  51908 + DOY        2016  57387 + DOY

2002  52274 + DOY        2017  57753 + DOY

2003  52639 + DOY        2018  58118 + DOY

2004  53004 + DOY        2019  58483 + DOY

2005  53370 + DOY        2020  58848 + DOY